Part 1:

Packet Tracer uses a different link light on one of the connections between the switches. What do you think this link light means?

- The orange link light indicates that the port is in the Blocking state (S1's G0/2 port connecting to S2). The link is physically up, but the port does not forward or receive data frames, in order to prevent a switching loop.

What path will frames take from PC1 to PC2?

- Frames travel from PC1 → S1 → S3 → S2 → PC2, going through S3 instead of the direct S1-S2 link.

Why do the frames not travel through S3?

(Note: based on the topology confirmed, frames actually DO travel through S3 — the direct S1-S2 link is the one being blocked. If your lab's specific question/topology differs, let me know and I'll adjust.)

- The direct link between S1 and S2 is blocked by Spanning Tree Protocol, so frames must take the alternate path through S3 instead.

Why has spanning tree placed a port in blocking state?

- Because S1, S2, and S3 are connected in a loop (triangle topology). If all links were forwarding, frames would circulate endlessly, causing a broadcast storm and MAC table instability. STP blocks one port to break the loop while keeping the link available as a backup path.

Part 2:

What do you see happen to the status of the G0/2 port during this process?

- The port transitions through the STP states in order: Blocking → Listening → Learning → Forwarding, taking approximately 30-50 seconds total (Listening 15s + Learning 15s by default) before it becomes fully forwarding.

Are any ports showing an orange link light that indicates that the port is in a spanning-tree state other than forwarding? Why or why not?

- No, after removing the S1-S2 link, no ports show an orange light. Since the loop no longer exists (only one path remains: S1-S3-S2), STP no longer needs to block any port, so all ports transition to the forwarding state.