Packet Tracer - Verify Single-Area OSPFv2

### Part 1: Verify the Existing OSPFv2 Network Operation

#### Step 1 – R1

Q: How did router R1 receive the default route?
 A: R1 received it via OSPF, shown as O*E2 (OSPF External Type 2) in the routing table — meaning the default route was redistributed into OSPF from an external source, not originated within OSPF itself.

Q: From which router did R1 receive the default route?
 A: From R2 — the next hop is 172.16.3.2 (R2's S0/0/0 address), reached via Serial0/0/0.

Q: How can you filter the output of show ip route to show only routes learned through OSPF?
 A: Use the command:

R1# show ip route ospf

Q: Which routers have formed adjacencies with router R1?
 A: R2 (Router ID 2.2.2.2, via Serial0/0/0) and R3 (Router ID 3.3.3.3, via Serial0/0/1).

Q: What are the router IDs and state of the routers shown in the command output?
 A: Both neighbors show State = FULL, with Priority = 0 and no DR/BDR displayed (FULL/ -), because both links are Serial point-to-point connections.

Q: Are all of the adjacent routers shown in the output?
 A: Yes — R1 is physically connected only to R2 and R3 via OSPF-enabled interfaces (its G0/1 link to the ISP is not part of OSPF), so both expected neighbors are present.

Q: Ping the ISP Router from PC1 — is it successful?
 A: Yes, successful (3 of 4 replies received; the first packet timed out due to normal ARP resolution delay, not an OSPF issue).

#### Step 2 – R2

Q: How did router R2 learn the default route to the ISP?
 A: Via a static route (S* 0.0.0.0/0), configured to point directly to the ISP through G0/1. R2 then redistributes this static route into OSPF, which is why R1 sees it as an O*E2 route.

Q: What type of OSPF network is attached to interface G0/0?
 A: BROADCAST network type.

Q: Are OSPF hello packets being sent out this interface? Explain.
 A: No — the interface is configured as a passive-interface. The network is still advertised into OSPF, but no Hello packets are sent or received on G0/0, so no neighbor adjacency can form there (confirmed by Neighbor Count = 0). This is because G0/0 connects to a LAN (PC2), not to another router.

Q: Ping the S0/0/1 address on R3 from PC2 — is it successful?
 A: Yes, successful (4/4 packets received, 0% loss).

#### Step 3 – R3

Q: Router R3 is routing for which networks?
 A: 192.168.1.0/24, 192.168.10.4/30, and 192.168.10.8/30 (all in Area 0).

Q: What is the neighbor priority shown for the OSPF neighbor routers? Is this the default?
 A: Priority = 0 for both neighbors (R1 and R2). This is the default value on point-to-point Serial links, since no DR/BDR election occurs on that network type.

Q: Ping the ISP Router from PC3 — is it successful?
 A: Yes, successful (4/4 packets received, 0% loss).

### Part 2: Add the New Branch Office LAN to the OSPFv2 Network

#### Step 1 – Verify R4 Configuration

Q: Which interface is configured to not send OSPF update packets?
 A: GigabitEthernet0/0/1 — configured with passive-interface GigabitEthernet0/0/1, since it connects to the Branch Office LAN (Access Point/Laptop) and does not need to form OSPF neighbors there.

#### Step 2 – Connect R4 to the OSPFv2 Network

Q: What state is displayed for router R3 (as seen from R4)?
 A: FULL/DR — R3 is the Designated Router on the shared Ethernet segment, and R4 (as BDR) is fully adjacent with it.

Q: Why is the state of router R4 different from the state of R1 and R2 (as seen from R3)?
 A: Because the R3–R4 link is an Ethernet (Broadcast) network, which requires a DR/BDR election — R3 became DR and R4 became BDR, so the state shows as FULL/BDR. In contrast, R3's links to R1 and R2 are Serial point-to-point connections, which do not use DR/BDR elections, so those neighbors show simply as FULL/ -.

Q: Ping PC2 from the Laptop — is it successful?
 A: Yes, successful (4/4 packets received, 0% loss), confirming that the new Branch Office LAN (192.168.11.0/24) has been successfully integrated into the OSPFv2 network and is reachable end-to-end.