# Packet Tracer - WAN Concepts — Answer Sheet (Completed)

Part 1: Investigate Consumer WAN Technologies for Home and Mobile Devices

Step 1(a) — What are the WAN technologies in use?
 The home networks use cable and DSL WAN technologies.

Step 1(b) — What media is used to connect the two home networks to the ISP? What devices are directly connected to the ISP?
 The Cable network uses coaxial media, connecting to a coaxial splitter device. The DSL network uses phone cable, connecting to a DSL modem.

Step 1(c) — What ports are available on the DSL modem and what is connected to them?
 The DSL modem has two ports: one connects to the phone line from the Telco, and the other connects to the home LAN over Ethernet.

Step 1(c) — What is the purpose of the DSL modem?
 It converts the telephone data network signals to Ethernet for the home network.

Step 1(c) — What is the type of connection between the ISP/Telco/Cable Company network and the Home Cable Network? Why is the splitter necessary?
 The connection is made with coaxial cable. The splitter is necessary because the cable carries both digital data and video signals — it separates the data signal (to the cable modem) from the video signal (to the TV).

Step 1(d) — What does the cable modem do? What connections does it have?
 It converts cable data signals to Ethernet signals. It connects to coaxial cable from the splitter and UTP cable via its Ethernet interface.

Step 1(d) — Which port does the cable from the cable modem connect to on the home wireless router? Where did the interface IP address come from?
 It connects to the internet interface, with the IP address obtained from the ISP network via DHCP.

Step 1(e) — What is the smartphone's IP address? Where did it come from?
 198.51.100.100, obtained from the Telco network via DHCP.

Step 1(e) — What data service is the cellphone currently using?
 Cellular data (3G/4G network).

Step 2: Explore the Business WAN

Step 2(a) — What different types of connections are used in the Business network?
 Ethernet over copper and fiber, plus serial connections.

Step 2(b) — What types of interfaces are present on the StoreNet switch?
 Gigabit Ethernet copper media ports and four modular ports with three GLC-LH-SMD fiber optic SFP modules installed, enabling fiber optic Ethernet connectivity.

Step 2(b) — Which interfaces and media connect the store networks to Business Headquarters? Why?
 Ethernet over fiber optic cable connects the stores to the StoreNet switch — chosen because of the distance required to reach the stores (a real deployment would use a provider's fiber service; simplified here for the activity).

Step 2(b) — What type of WAN service connects the Business Headquarters router to the ISP?
 A serial WAN connection.

Part 2: Explore Connectivity

Can all hosts ping each other and the web server?
 Hosts in both networks can ping the web server, but hosts on the Business WAN cannot ping hosts on the Consumer WAN, and vice versa.

Is this a good situation?
 Yes — these networks should not be directly reachable from outside each other for security reasons.

Key Ping Results from the Activity

| Ping Test | Result |
| --- | --- |
| PC0 192.168.10.2 → Store 2-1 192.168.20.3 | Successful |
| PC0 192.168.10.2 → Web Server 209.165.200.226 | Successful |
| Laptop 2 192.168.0.100 → Laptop 1 192.168.0.102 | Successful |
| Laptop 2 192.168.0.100 → PC0 192.168.10.2 | Unsuccessful / Destination host unreachable |
| Laptop 2 192.168.0.100 → Web Server 209.165.200.226 | Successful |